CalEEMod Training Example Project – Wed. February 29, 2012 at MTC/ABAG Auditorium, 101 8th St. Oakland

TITLE:  California Mixed Use Complex;  Air District:  BAAQMD; Climate Zone 5; Urban; Op Yr 2012; PG&E

|  |  | LECTURING NOTES |
| --- | --- | --- |
| Residential –Mid-Rise Apartments (2nd-5th floors) | 100 Dwelling Units (1.6 acres) | ITE trip rate = low rise apt. that is baseline for CAPCOA GHG mitigation measures so user can apply |
| Retail – Supermarket (1st floor) | 45,000 sq ft | Zero out acres since same footprint as residential |
| Recreational – Quality Restaurant | 5,000 sq ft | Mixed use credit can be applied under Mitigation section |
| Parking – Parking Lot | 50,000 sq ft (1.15 acres) | New land use category – separate entry; more accurate calculation (assumes 400 sq ft/space) |
| Modify construction phase |  | Default phases can be changed – example, the developer wants to cut the schedule in half; changes will be highlighted in yellow reminding user to justify change under Remarks; hit “default” to re-populate with default values. |
| Demolition | 0 days | Changing # of days will automatically change END date; need to change start date, then # days. If using the CSV template, need to enter 4 digit year, 2 digit month, 2 digit day |
| Site Preparation | 2 days |  |
| Grading | 3 days |  |
| Building Construction | 110 days |  |
| Paving | 5 days |  |
| Architectural Coatings | 5 days |  |
| Remarks to justify change |  | Project proponent reduced the construction schedule in half the time as the defaults. |
| Construction Equipment |  | Defaults based on one crew of equipment from construction survey. If reducing construction schedule in half, should double equipment. (Remarks) Download data to individual screen by using “import CSV” and browse for csv file (appropriate tab from excel saved as CSV); NOTE: if using template, erase extra unused lines (down to 100). |
| Material exported – site preparation | 32 cu yds | User needs to input the amount of material imported and/or exported from the site; if phased, less trips. NOTE: Trips+VMT tab provides default worker, vendor and hauling trips and assumes hauling truck carries 16 cu yd |
| Material exported – grading | 16 cu yds |  |
| Operational – Water/Wastewater | 100% aerobic | Can change (statewide) default to reflect site specific wastewater treatment (Remarks) |
| Vegetation – grassland | 3.25 - 1 acres | Change initial to final acres; GHG reduction (annual only) |
| Sequestration – new Hardwood Maples | 14 trees | Will provide GHG benefit |
| Construction mitigation - engine | Tier 3 | All equipment. Benefit from default fleet mix. (Remarks) |
| Construction mitigation – watering | 2 times/day | Automatic default percent reduction |
| Project setting | Urban | Dictates the allowable percent reduction (and cap) |
| Increase density | 36 DU/acre | # DU/total project acre developed (100/2.75) |
| Increase diversity | Yes | Check box if mixed use project |
| Improve destination accessibility | 0.2 | Project in center city so not far to walk to work |
| Improve transit accessibility | 0.01 | Transit-oriented development in front of building |
| Integrate below market rate | 10 | For example: if 10% required by local law (10% of 100=10) |
| Improve pedestrian network | Project site and connecting offsite |  |
| Implement NEV network | Applies low penetration |  |
| Area | No hearths |  |
| Energy – exceed Title 24 | 15% (LEED status) |  |
| Alternative Energy | Check “on-site renewable energy” to account for 8,390 kW from solar panels on roof (medium size system) |  |